Analog Devices SPICE Macro-model Library     Release C, 1/91

Copyright 1991 by Analog Devices, Inc.

LICENSE STATEMENT

The information on this diskette is protected under the

United States copyright laws.  Analog Devices, Inc.("ADI")

hereby grants users of these macro-models hereto referred

to as licensee, a nonexclusive, nontransferable license to

use these macro-models as long as the licensee abides by

the terms of this agreement.  Before using the macro-

models the licensee should read this license.  If the

licensee does not accept these terms, this diskette should

be returned to ADI within 30 days.

The licensee may not sell, load, rent, lease, or license

the macro-models, in whole, in part, or in modified form,

to anyone outside the licensee's company.  The licensee

may modify these macro-models to suit his specific

applications, and the licensee may make copies of this

diskette or macromodels for use within his company only.

These macro-models are provided "AS IS, WHERE IS, AND WITH

NO WARRANTY OF ANY KIND EITHER EXPRESSED OR IMPLIED,

INCLUDING BUT NOT LIMITED TO ANY IMPLIED WARRANTIES OF

MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE."

In no event will ADI be liable for special, collateral,

incidental, or consequential damages in connection with or

arising out of the use of these macro-models.  ADI

reserves the right to make changes to the products and the

macro-models without prior notice.

APPLICATION INFORMATION

The SPICE macro-models on this diskette, as of the above

release date, are the most current for Analog Devices and

PMI's products.  As more models are completed, new

releases of this diskette will be issued.

These models have an advanced architecture that allows

for multiple poles and zeros to accurately model the AC

and transient response of the device.  Unique to ADI, the

following models contain noise sources like the real op

amp, allowing SPICE analysis of the total noise of a

circuit:

OP-27

OP-37

OP-77

OP-177

Also included are two instrumentation amplifier models:

AMP-01

AMP-02

This diskette contains macro-model net-lists only.  It

does NOT contain actual SPICE simulator software.  The

user must obtain their own SPICE simulator.  All the

model net-lists conform to the standard SPICE format and

can be run on most SPICE based simulators.  An example

circuit net-list is included in the file "EXAMPLE.CIR",

which can be run to check the models on a specific

simulator.

For more information on the SPICE models please contact

the ADI literature department and request the following

application notes:

AN110, AN117, AN120, AN126, AN132, and AN138

Write:   Analog Devices, Inc.

Literature Dept.

One Technology Way

P.O. BOX   9106

Norwood, MA 02062      USA

MODEL REVISIONS

Some of the models have been revised since Release B of

the diskette.  If a model was revised, the release letter

on the first line of the file was changed, and the

revisions are documented in the comments at the beginning

of the net-list file.  IMPORTANT:  THE ORDER OF THE NODE

CALL-OUTS HAS CHANGED TO PUT THE OUTPUT NODE LAST.  In

the previous releases the output node was always third,

but now it is last after the power supplies, to agree

with the standard SPICE op amp model call-out.

OPEN-LOOP GAIN AND PHASE

A common mistake in simulating an amplifier's open-loop

gain and phase performance is to run the model open-loop.

Like the actual device, without feedback, even a small

amount of offset can cause the circuit to saturate to

either supply rail.  Because of the saturation, the model

gives a much lower open-loop gain.  Thus the model needs

to be run closed-loop.  Typically the easiest way to do

this is to run the model in an inverting op amp

configuration and have SPICE plot the log of the output

minus the log of the inverting input.

TRANSIENT ANALYSIS

One of the most difficult problems in using SPICE is

convergence.  As the number of nodes increases, so do the

calculations needed for convergence.  If the analysis

quits due to non-convergence then adding the following

line to the circuit net-list will help in most cases:

.OPTIONS  ITL4=40

This increases the number of iterations from the default

of 10 to 40.  SPICE simulator will then make 40

iterations to try to find the next point of the transient

solution before going to a smaller time step.

NEGATIVE CAPACITANCE

Some SPICE simulators do not allow the use of a negative

capacitor, which is used in some macro-models to properly

shape the frequency response.  Those simulators will

issue an error message stating that the negative

capacitor is not permissible.  When this problem is

encountered, change the sign of the negative capacitor to

positive.  Then, change the signs of the two resistors in

the same stage to negative.

POWER DISSIPATION

All the models accurately model the supply current of the

part including the output current.  Thus to calculate the

power dissipation of the circuit, the supply current

should be multiplied by the supply voltage.  This may

give a different number compared to the total power

dissipation printed at the end of all SPICE output files.

This is due to higher than normal internal currents,

which are not reflected in the supplies.  The noise

models are especially affected by this because of the

large currents needed to keep the noise low.  Thus,

always use the actual supply currents to calculate the

total power dissipation.